# Autonomous BSP Validation Framework - Test Suite Details

This document outlines all hardware peripheral validation scripts and their constituent test cases. The framework is inherently board-agnostic and designed to discover and validate any Linux-capable target device dynamically. All hardware-specific setups have been abstracted.

# test_ethernet.py

Validates functionality related to ethernet.

## test_ethernet_interface_state

Validates that the Ethernet interface is physically up and active.
Checks the /sys/class/net/ethX/operstate file.

## test_ethernet_ip_check

Validates that the interface has an assigned IPv4 address.

## test_ethernet_ping_connectivity

Validates external connectivity by pinging a known target.

## test_ethernet_latency

Validates network delay/latency is within acceptable thresholds.
Extracts the 'avg' rtt from a ping.

## test_ethernet_socket

Validates local networking stack by opening a raw TCP socket.

## test_ethernet_speedtest

Measures Internet bandwidth using speedtest-cli.
Requires speedtest-cli to be installed via apt.

## test_ethernet_iperf

Measures local network professional throughput using iperf3.
Requires an iperf3 server running on the target network.

# test_gpio.py

Validates functionality related to gpio.

## test_gpio_high_low

Test basic HIGH/LOW propagation with visual LED pauses.
🔌 The Hardware Setup
Connect a single jumper wire directly from your configured out_pin 17 to your configured in_pin 27.

## test_gpio_pull_up_down

Test internal pull-up and pull-down resistors with observation pauses.
🔌 The Hardware Setup
Maintain the exact same baseline setup as the previous test (Target Devicen 17 ➔ Target Devicen 27), 
but for this test we will leave the OUT pin disconnected (High-Z) to allow the internal 
pull resistors to do their job without interference.

## test_gpio_edge_interrupts

Interactive test to physically confirm hardware interrupts.
    🔌 The Hardware Setup
Maintain the exact same baseline setup as the previous test (Target Devicen 17 ➔ Target Devicen 27), 
but for this test we will be generating electrical edges on the OUT pin and confirming 
that the IN pin triggers hardware interrupts in the kernel.

## test_gpio_level_polling

Simulates Level-Triggered validation via high-frequency polling.
        🔌 The Hardware Setup
Maintain the exact same baseline setup as the previous test (Target Devicen 17 ➔ Target Devicen 27)

## test_gpio_level_polling_low

Simulates Level-Triggered validation for Active-LOW signals.
🔌 The Hardware Setup
Maintain the exact same baseline setup as the previous test (Target Devicen 17 ➔ Target Devicen 27)

## test_gpio_muxing_state

Validates that the SoC's internal router (Target Devicenmux) has correctly assigned a pin.
Because this test strictly queries the internal Peripheral Controller silicon router and does not measure 
external electrical thresholds, no physical wiring is required. The test runs entirely inside the chip's logic gates.

## test_gpio_drive_strength_basic

Interactive test to validate output drive strength under physical load.
🔌 The Hardware Setup
Maintain the exact same baseline setup as the previous test (Target Devicen 17 ➔ Target Devicen 27)

# test_i2c.py

Validates functionality related to i2c.

## test_i2c_bus_detection

Validates that the target I2C bus is enabled and recognized by the OS.

## test_i2c_virtual_device_scan

Tests the parsing logic against an automated Software-In-Loop (SIL) environment.

## test_i2c_nack_error_handling

Validates that addressing a non-existent device properly catches a NACK.

# test_leds.py

Validates functionality related to leds.

## test_status_led_sysfs_control

Validates onboard status LED control via sysfs:
  1. Enumerates all available LEDs
  2. Tests each LED: manual ON/OFF toggle
  3. Tests timer (blink) trigger
  4. Restores original state for every LED

## test_physical_button_interrupts

Interactive test to physically validate external button interrupts.

## test_physical_button_debounce

Interactive test to prove the kernel debounce filter stops mechanical bounce.

# test_pcie_hardware.py

Validates functionality related to pcie_hardware.

## test_pcie_device_present

1. PCIe Enumeration Test
Verify endpoint is visible.

## test_link_speed

2. Link Speed Validation
Target Device typically negotiates: PCIe Gen2/Gen3 x1.

## test_link_width

3. Link Width Validation

## test_driver_loaded

4. Driver Binding Test
Verify kernel driver loaded.

## test_bar_assignment

5. BAR Assignment Test

## test_no_pcie_errors

6. Kernel Error Scan
Critical BSP validation test.

## test_nvme_detected

7. NVMe Presence Test
If NVMe SSD attached.

## test_nvme_rw

8. NVMe Read/Write Test
Create temporary file dynamically on the mounted NVMe drive.

## test_nvme_throughput

9. FIO Throughput Test
Useful for BSP performance baselining.

## test_interrupts

10. Interrupt Validation
Verify MSI interrupts increase specifically for the NVMe controller.

## test_remove_rescan

11. Hot Reset Test
PCIe recovery validation.

## test_post_reboot

12. Reboot Persistence Test
Run after reboot: (Verifies it is currently visible)

## test_24hr_stability

13. Long-Duration Stability Test
Continuous stress: (Shortened for standard CI runs)

# test_pcie_software.py

Validates functionality related to pcie_software.

## test_internal_device_present

Validates that the internal PCIe bridge and Southbridge chip are present on the bus.

## test_internal_link_speed

Validates the negotiated PCIe link speed is optimal (e.g. 5.0GT/s).

## test_internal_link_width

Validates the negotiated PCIe link width is at maximum capacity (e.g. x4 lanes).

## test_internal_driver_loaded

Verifies that the correct kernel driver is loaded and bound to the PCIe endpoint.

## test_internal_bar_assignment

Checks that Memory Base Address Registers (BARs) are correctly assigned by the kernel.

## test_internal_no_pcie_errors

Scans the kernel dmesg ring buffer to ensure no critical PCIe errors (like AER or CRC) occurred.

## test_rp1_peripherals_detected

Verifies that internal peripherals downstream of the PCIe bridge (like Ethernet and GPIO) enumerate successfully.

## test_rp1_data_transfer

Reads a hardware register to confirm data transfer across the internal PCIe link.

## test_rp1_throughput

Simulates memory subsystem stress to validate internal PCIe bridge responsiveness under load.

## test_internal_interrupts

Polls hardware state to ensure PCIe interrupts are correctly firing and counted by the kernel.

## test_internal_remove_stage1

Initiates a hot-reset of the PCIe Southbridge to validate kernel panic recovery and reboot persistence.

## test_internal_post_reboot

Validates that the internal PCIe endpoint recovers successfully after a hot-reset reboot.

## test_internal_24hr_stability

Continuously polls hardware sensors over a duration to ensure long-term link stability.

# test_rtc.py

Validates functionality related to rtc.

## test_rtc_stage1_hardware_node

Stage 1: Verify RTC device node exists, is accessible,
and the correct driver is loaded.

## test_rtc_stage2_drift_check

Stage 2: Read hardware clock and measure drift against system clock.
Validates RTC is ticking and within acceptable tolerance.
NOTE: This does NOT prove retention — it proves the RTC is running now.

## test_rtc_stage3_pre_reboot_stamp

Stage 3: Write a timestamp to the RTC and to a state file.
Run this BEFORE rebooting. After reboot, run Stage 4 to verify retention.

WORKFLOW:
  1. Run this test  → writes timestamp, reboots Target Device
  2. After reboot, run test_rtc_stage4_post_reboot_verify

## test_rtc_stage4_post_reboot_verify

Stage 4: Read RTC after reboot and compare against pre-reboot stamp.
MUST be run in a SEPARATE pytest session AFTER rebooting following Stage 3.

## test_rtc_battery_power_off_retention

Stage 3: Validates the physical RTC battery retained the clock state 
while main power was completely severed.
Requires the 'Fake Time Trap' to be set prior to execution.

## test_ntp_sync_interaction

Validates NTP sync interaction with the RTC:
  1. NTP service is active
  2. System clock synchronizes to network time
  3. NTP offset is within acceptable bounds
  4. Synchronized time is written back to RTC (NTP→RTC interaction)
  5. RTC matches NTP-corrected system time

# test_spi.py

Validates functionality related to spi.

## test_spi_master_loopback

Validates SPI Master mode across all 4 CPOL/CPHA combinations.

## test_spi_chip_select_assertion

Validates the full Chip Select (CS) lifecycle using kernel edge-buffers.

## test_spi_clock_polarity_and_phase

Software logic analyzer for CPOL/CPHA validation.

## test_spi_data_integrity_stress

Validates SPI data integrity by blasting edge-case patterns
and maximum-buffer payloads at 10 MHz across all 4 SPI modes.

# test_timer.py

Validates functionality related to timer.

## test_timer_jitter

Validates the hardware/OS timer precision by checking for sleep jitter.
Expects standard 10ms sleeps to complete within an acceptable margin.

## test_timer_stress

Spawns multiple concurrent timers to ensure the system scheduler
and timer queues handle high concurrency gracefully.

## test_timer_cron

Validates that the cron daemon is actively monitoring the system clock
and dispatching tasks at the correct minute boundaries.

# test_uart.py

Validates functionality related to uart.

## test_uart_physical_loopback

Validates UART TX/RX hardware via physical loopback.
Tests multiple baud rates, edge-case payloads, and framing/error flags.

## test_uart_baud_rate_sweep

Validates the hardware baud-rate generator by sweeping legacy,
standard, and high-speed UART configurations via physical loopback.

## test_uart_hardware_flow_control

Validates RTS/CTS Hardware Flow Control via software-driven CTS toggling.

## test_uart_console_vs_secondary_isolation

Validates Console UART vs Secondary UART isolation.

# test_watchdog.py

Validates functionality related to watchdog.

## test_watchdog_enable_and_feed

Validates hardware watchdog: enable, configure, feed, graceful disarm.
No wiring required — purely internal silicon.

Proves:
  1. /dev/watchdog0 opens and configures correctly
  2. Timeout readback matches requested value
  3. KEEPALIVE IOCTL succeeds on every cycle
  4. Magic 'V' disarms cleanly (no reboot triggered)

## test_watchdog_starvation_reboot

DESTRUCTIVE TEST: Proves that failing to feed the watchdog triggers a reboot.

TWO-PHASE WORKFLOW:
  Phase 1 (this run):   Arm watchdog, close WITHOUT 'V' → Target Device reboots
  Phase 2 (after reboot): Run test_watchdog_post_reboot_verify to confirm
                           the reboot was caused by the watchdog.

Safety guard: set 'allow_destructive_reboot: True' in board_config to run.

## test_watchdog_post_reboot_verify

Phase 2 of starvation test — run AFTER the Target Device reboots.
Reads WDIOC_GETBOOTSTATUS to confirm the reboot was watchdog-caused.